CVS Health Information Security

IDS/IPS & DLP Policy State Assessment

Firewall Estate — 183 Devices (86 ASA, 97 FTD/Snort)  |  Generated: June 29, 2026

# 1. Executive Summary

The CVS Health firewall estate comprises 183 unique devices across 12 functional segments. This assessment evaluates the deployed state of intrusion detection/prevention (IDS/IPS) and data loss prevention (DLP) controls derived directly from firewall running configurations. Overall estate security posture scores 42/100 — rated High risk.

| Control Domain | Devices Covered | Devices Lacking | Coverage % | Rating |
| --- | --- | --- | --- | --- |
| IPS — Active/Licensed | 19 | 164 | — | — |
| IPS — Engine Present (incl. Snort w/o license) | 110 | 73 | — | — |
| Threat Detection (basic/Snort) | 134 | 49 | — | — |
| DLP / Malware / File Inspection | 2 | 181 | — | — |
| Logging Enabled | 148 | 35 | — | — |
| NetFlow / Traffic Telemetry | 10 | 173 | — | — |
| TLS 1.2+ (management plane) | 27 ASA + 97 FTD | 17 ASA on TLS 1.0 | — | — |
| Anti-Spoofing (uRPF) | 19 | 164 | — | — |

Key findings:

- IDS/IPS: 97 FTD devices carry a Snort3 engine but only 6 are IPS-licensed (90 run ESSENTIALS-only with no IPS enforcement). 13 ASA devices have a hardware IPS module — all on RI-One and Windsor VPN concentrators and perimeter ASAs. 49 ASA devices have neither an IPS module nor threat-detection enabled.

- DLP: File inspection / malware defense exists on only 2 devices — the Richmond SAP cluster (RVAASCP). No DLP capability is deployed on any 3rd-party connection segment, BPO MPLS, cloud extranet, or VPN concentrator.

- SSL/TLS: 17 ASA devices still negotiate TLS 1.0 on their management/VPN plane. 42 ASA devices have no SSL version configured at all.

- Logging: 35 ASA devices have logging disabled — including 13 RI-One perimeter firewalls and all 8 PDC DMZ/service firewalls.

- NetFlow: Only 10 devices export flow telemetry — limited to VPN concentrators (167.69.34.237) and a small number of other devices. No BPO, MPLS, or cloud extranet segments have flow export.

# 2. Security Posture Scorecard by Segment

Score is a weighted composite: IPS Active (25 pts), TLS 1.2+ (15 pts), Logging (20 pts), Threat Detection (15 pts), NetFlow (10 pts), Anti-Spoofing (10 pts), DLP/AMP (5 pts).

| Category | Devices | Score | Risk | IPS Active | Threat Det | Logging | TLS 1.2+ | NetFlow | DLP/AMP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CME/CMC GLR | 2 | 0/100 | Critical | 0% | 0% | 0% | 0% | 0% | 0% |
| RI-One Firewalls | 15 | 10/100 | Critical | 13% | 13% | 13% | 0% | 7% | 0% |
| PDC Firewalls | 13 | 15/100 | Critical | 0% | 38% | 38% | 0% | 8% | 0% |
| Windsor Firewalls | 9 | 21/100 | Critical | 22% | 33% | 33% | 0% | 11% | 0% |
| NET4WIR ASA | 13 | 37/100 | High | 0% | 0% | 85% | 85% | 0% | 0% |
| Shea DC Firewalls | 21 | 44/100 | High | 14% | 76% | 90% | 67% | 5% | 0% |
| Cloud Extranet FTD | 33 | 50/100 | Medium | 0% | 100% | 100% | 97% | 0% | 0% |
| MPLS-East FTD | 23 | 50/100 | Medium | 0% | 100% | 100% | 100% | 0% | 0% |
| MPLS-West FTD | 27 | 50/100 | Medium | 0% | 100% | 100% | 100% | 0% | 0% |
| Other | 10 | 55/100 | Medium | 50% | 90% | 90% | 60% | 10% | 0% |
| VPN Concentrators | 14 | 57/100 | Medium | 36% | 93% | 93% | 64% | 36% | 0% |
| Richmond (SAP) | 3 | 65/100 | Medium | 67% | 100% | 100% | 67% | 0% | 67% |
| OVERALL | 183 | 42/100 | High | 10% | 96% | 81% | 68% | 5% | 1% |

Risk Rating Legend:

|  | Critical (0–29) | Fundamental controls absent: no IPS, no logging, no TLS1.2. Immediate remediation required. |
| --- | --- | --- |
|  | High (30–49) | Major gaps in IPS or visibility. Significant risk exposure on traffic traversing this segment. |
|  | Medium (50–69) | Partial controls. Snort/logging present but IPS not licensed or threat-detection absent. |
|  | Low (70–100) | Core controls in place. Fine-tuning and advanced capabilities (AMP/DLP) still applicable. |

# 3. IDS/IPS Policy State

## 3.1  Architecture Overview

The estate uses four distinct IPS/IDS mechanisms depending on platform generation:

| Mechanism | Platform | Devices | Mode | Enforcement Capability |
| --- | --- | --- | --- | --- |
| Cisco ASA Hardware IPS Module | ASA (legacy) | 5 VPN concentrators + 8 perimeter ASAs | Inline (if configured) Promiscuous available | Full signature-based IPS/IDS — Snort signature database on dedicated blade. Mode (inline/promiscuous |
| Cisco FTD Snort 3 Engine | FTD | 97 devices total | Inline (always) | IPS enforcement requires IPS license. Without license: Snort runs but rules are in monitor/disabled |
| ASA Threat-Detection | ASA | 59 devices | Passive statistical | Rate-based DoS/scanning detection only. No protocol signatures. Cannot detect exploit traffic — stat |
| ASA MPF Application Inspection | ASA | All ASA devices | Inline (protocol only) | Protocol conformance enforcement (malformed packet drops, RFC compliance). NOT signature IPS — no CV |

## 3.2  IPS Coverage by Segment

Percentage of devices with active IPS enforcement (IPS-licensed FTD or hardware IPS module on ASA):

| Segment | Devices | IPS Active | IPS Pct | Threat Det | Platform Note |
| --- | --- | --- | --- | --- | --- |
| CME/CMC GLR | 2 | — | — | — | No IPS control |
| Cloud Extranet FTD | 33 | — | — | — | Snort3 present; ESSENTIALS license only — no IPS enforcement |
| MPLS-East FTD | 23 | — | — | — | Snort3 present; ESSENTIALS license only — no IPS enforcement |
| MPLS-West FTD | 27 | — | — | — | Snort3 present; ESSENTIALS license only — no IPS enforcement |
| NET4WIR ASA | 13 | — | — | — | No IPS control |
| PDC Firewalls | 13 | — | — | — | No IPS control |
| RI-One Firewalls | 15 | — | — | — | No IPS control |
| Shea DC Firewalls | 21 | — | — | — | No IPS control |
| Windsor Firewalls | 9 | — | — | — | No IPS control |
| VPN Concentrators | 14 | — | — | — | HW IPS module on mid/win VPNs only |
| Other | 10 | — | — | — | No IPS control |
| Richmond (SAP) | 3 | — | — | — | IPS + MALWARE_DEFENSE licensed |

## 3.3  FTD IPS Licensing Gap

90 of 97 FTD devices carry ESSENTIALS license only. Snort3 is present on all of them (engine version SNORT3), but without the IPS entitlement the access policy rules operate in:

- Detect-Only mode — traffic matches rules and events are logged but sessions are NOT dropped.

- The seven IPS-licensed FTD devices are: PAZSHDC-SEC-EXT-CTE-01, PAZSHDC-SEC-INTEXP-01, RRIWSCP-SEC-EXT-DCOM-01, RRIWSCP-SEC-EXT-DCOM-02, RVAASCP-SEC-SAP-01, RVAASCP-SEC-SAP-02.

- The remaining 90 devices — all BPO MPLS connections, cloud extranet (Ashburn/Dallas), and NET4WIR contexts — have Snort running but are detection-only.

## 3.4  ASA IPS Module Deployment

ASA hardware IPS modules are present on the following devices (IPS blade installed — mode inline vs. promiscuous requires live device verification):

| Device | Category | IPS Mode in Config | Notes |
| --- | --- | --- | --- |
| mid-fw-siptest | RI-One Firewalls | Not specified in config | MDC perimeter — inspect DMZ and external traffic |
| PAZSHDC-SEC-INT-TAP-01 | Shea DC Firewalls | Not specified in config | Perimeter device |
| PRIWSDRSECEXTADMVPN-01 | VPN Concentrators | Not specified in config | B2B VPN traffic — highest value target for IPS inspection |
| RRIWNCP-SEC-MDF-PRINT-01 | Other | Not specified in config | Perimeter device |
| RRIWSDC-SEC-EXT-VPN-01 | Other | Not specified in config | Perimeter device |
| RRIWSDC-SEC-INT-VPN-01 | Other | Not specified in config | Perimeter device |
| win-fw-siptest | Windsor Firewalls | Not specified in config | Perimeter device |
| mid-fw-edmz | RI-One Firewalls | Not specified in config | MDC perimeter — inspect DMZ and external traffic |
| mid-fw-vpn1 | VPN Concentrators | Not specified in config | B2B VPN traffic — highest value target for IPS inspection |
| mid-fw-vpn2 | VPN Concentrators | Not specified in config | B2B VPN traffic — highest value target for IPS inspection |
| win-fw-edmz | Windsor Firewalls | Not specified in config | Perimeter device |
| win-fw-vpn1 | VPN Concentrators | Not specified in config | B2B VPN traffic — highest value target for IPS inspection |
| win-fw-vpn2 | VPN Concentrators | Not specified in config | B2B VPN traffic — highest value target for IPS inspection |

# 4. DLP Policy State

Data Loss Prevention in the Cisco firewall estate is implemented through two mechanisms: (1) FTD Malware Defense (AMP) with file inspection, and (2) FTD SSL decryption for inspecting encrypted data flows. Neither capability requires the base ESSENTIALS license — both require add-on entitlements.

| DLP Capability | Requires | Devices Deployed | Devices Missing | Coverage |
| --- | --- | --- | --- | --- |
| File Inspection (AMP/Malware) | MALWARE_DEFENSE license | RVAASCP SAP cluster (2 devices) | 181 of 183 | — |
| URL / Category Filtering | URL license | RVAASCP SAP cluster (2 devices) | 181 of 183 | — |
| SSL/TLS Decryption | SSL license + cert deployment | None detected in configs | All ${est.total} devices | — |
| ASA DLP (native) | N/A — not supported | None | All ${est.asa} ASA devices | — |

Assessment:

- DLP coverage across the CVS Health firewall estate is effectively zero outside the Richmond SAP cluster. No 3rd-party connection segment, BPO MPLS path, cloud extranet connection, or VPN concentrator has file inspection or URL filtering capability.

- The Richmond SAP cluster (RVAASCP-SEC-SAP-01/02) is the sole exception — licensed for IPS + MALWARE_DEFENSE + URL. This is appropriate given it handles SAP application traffic but is not representative of the estate.

- ASA does not natively support DLP. Any DLP enforcement on ASA-terminated connections requires an upstream or inline DLP appliance (e.g., Symantec DLP, Forcepoint) not reflected in these configs.

- FTD TLS inspection (SSL decrypt-and-reinspect) is not configured on any device. Without TLS inspection, all HTTPS/encrypted traffic traverses these firewalls without content inspection regardless of IPS license state.

# 5. SSL/TLS & Management Plane Security

17 ASA devices still negotiate TLS 1.0, which is deprecated (RFC 8996, March 2021) and prohibited under PCI-DSS v4.0. 42 ASA devices have no SSL version configured, meaning they inherit Cisco's default (platform-dependent, often TLS 1.0).

| SSL/TLS Version | ASA Devices | Assessment |
| --- | --- | --- |
| TLS 1.2 only (tlsv1.2) | 27 | Compliant |
| TLS 1.0 + DTLS 1.0 (tlsv1 dtlsv1) | 17 | NON-COMPLIANT — PCI-DSS 4.0 violation |
| TLS 1.2 + DTLS 1.0 (tlsv1.2 dtlsv1) | 15 | Partial — DTLS 1.0 exposure |
| Not configured (default/unknown) | 42 | Unknown — verify per-device |
| FTD (FMC-managed, TLS config in FMC) | 97 | Not visible in exported config — verify in FMC |

Devices with TLS 1.0 enabled (management and VPN plane):

| Device | Category | SSL Version | Risk |
| --- | --- | --- | --- |
| PAZSHDC-SEC-EXT-NET4WIR-01 | NET4WIR ASA | tlsv1 dtlsv1 | — |
| test-text | NET4WIR ASA | tlsv1 dtlsv1 | — |
| mid-fw-edmz | RI-One Firewalls | tlsv1 dtlsv1 | — |
| mid-fw-vpn1 | VPN Concentrators | tlsv1 dtlsv1 | — |
| mid-fw-vpn2 | VPN Concentrators | tlsv1 dtlsv1 | — |
| DMVPN | VPN Concentrators | tlsv1 | — |
| RSIA | Shea DC Firewalls | tlsv1 | — |
| SIP | Shea DC Firewalls | tlsv1 | — |
| PAZSHDC-SEC-EXT-RS-01 | Shea DC Firewalls | tlsv1 | — |
| pdc-fw-aetna-1 | PDC Firewalls | tlsv1 dtlsv1 | — |
| pdc-fw-aetna-2 | PDC Firewalls | tlsv1 dtlsv1 | — |
| pdc-fw-cvty-1 | PDC Firewalls | tlsv1 dtlsv1 | — |
| pdc-fw-cvty-2 | PDC Firewalls | tlsv1 dtlsv1 | — |
| win-fw-edmz | Windsor Firewalls | tlsv1 dtlsv1 | — |
| win-fw-equinix-ftd-1-1 | Windsor Firewalls | tlsv1 dtlsv1 | — |
| win-fw-vpn1 | VPN Concentrators | tlsv1 dtlsv1 | — |
| win-fw-vpn2 | VPN Concentrators | tlsv1 dtlsv1 | — |

# 6. Logging & Traffic Visibility

| Visibility Control | Deployed | Missing | Coverage |
| --- | --- | --- | --- |
| Syslog enabled | 148 | 35 | — |
| NetFlow/flow export | 10 | 173 | — |
| Anti-spoofing (uRPF) | 19 | 164 | — |

## 6.1  Devices with Logging Disabled

35 ASA devices have logging disabled. All FTD devices log to FMC by default. Critical gaps:

| Segment | Device Count | Devices |
| --- | --- | --- |
| RI-One Firewalls | 13 | MID-FW-OFFICEEXT.config, mid-ds-l3-edmz-1_old, mid-fw-cvs-a_20260623_060120.config, mid-fw-cvs-activ |
| PDC Firewalls | 8 | pdc-fw-dmz-a.config, pdc-fw-dmz-b.config, pdc-fw-glr-a.config, pdc-fw-glr-b.config, pdc-fw-inetdmz-a |
| Windsor Firewalls | 6 | WIN-DS-L2-EDMZ-1_new, win-fw-cvs-a_20260617_062532.config, win-fw-cvs-active.config, win-fw-cvs-b_20 |
| Shea DC Firewalls | 2 | PAZSHDC-SEC-EXT-NET-PA-01;, PAZSHDC-SEC-EXTWIR-PA-01; |
| NET4WIR ASA | 2 | PAZSHDC-SEC-EXT-NET4WIR-01, test-text |
| CME/CMC GLR | 2 | cmc-fw-hubprodglr-1;, VM-FW1; |
| Other | 1 | PRIWSDCSECINT-PBM-RETAIL-01.config |
| VPN Concentrators | 1 | DMVPN |

## 6.2  NetFlow Export

NetFlow telemetry is exported by 10 devices to the following collectors:

| Collector IP | Exporting Devices | Likely Consumer |
| --- | --- | --- |
| 10.217.238.253 | 1 | Internal — verify Stealthwatch target |
| 204.99.37.30 | 1 | CVS/Caremark — verify Stealthwatch target |
| 10.220.239.216 | 1 | Internal — verify Stealthwatch target |
| 167.69.34.237 | 5 | Aetna/Internal — likely Stealthwatch/CSNA |
| 167.69.44.11 | 2 | Aetna/Internal — likely Stealthwatch/CSNA |

173 devices export no flow data — including all BPO MPLS, all cloud extranet, all NET4WIR contexts, and most perimeter firewalls. Without NetFlow, Stealthwatch/CSNA has no visibility into traffic patterns on these segments.

## 7. ASA Application Inspection Coverage

ASA Modular Policy Framework (MPF) inspects the following protocols for RFC conformance and protocol abuse. This is NOT signature IPS — it enforces protocol structure only.

| Protocol | ASA Devices | % of ASA estate | Classification |
| --- | --- | --- | --- |
| h323 | 78 | — | VoIP/media — call control inspection |
| ftp | 51 | — | Application layer — partial DLP value |
| tftp | 46 | — | Protocol conformance only |
| icmp | 46 | — | Protocol conformance only |
| sqlnet | 45 | — | Protocol conformance only |
| dns | 44 | — | DNS conformance — DNS tunneling detection limited |
| rsh | 44 | — | Protocol conformance only |
| sunrpc | 44 | — | Protocol conformance only |
| netbios | 44 | — | Protocol conformance only |
| ip-options | 44 | — | Protocol conformance only |
| xdmcp | 40 | — | Protocol conformance only |
| rtsp | 39 | — | VoIP/media — call control inspection |
| skinny | 39 | — | VoIP/media — call control inspection |
| sip | 37 | — | VoIP/media — call control inspection |
| esmtp | 30 | — | Application layer — partial DLP value |

Note: HTTP inspection covers 7 of 86 ASA devices. Without SSL inspection, HTTPS traffic (the majority of 3rd-party web traffic) is not inspected.

# 8. Recommendations

| # | Priority | Finding | Action |
| --- | --- | --- | --- |
| 1 | Critical | DLP gap: 181 of 183 devices have no file inspection or malware defense. No 3rd-party or BPO segment has any DLP capability. | Evaluate MALWARE_DEFENSE license deployment for FTD devices on highest-risk 3rd-party segments (NET4WIR, MPLSWEST/EAST BPO). For ASA-terminated connections, assess upstream Cisco DLP or Symantec DLP placement. PCI-scoped segments should be prioritized first. |
| 2 | Critical | 35 ASA devices have logging disabled. Includes all 8 PDC DMZ/service firewalls, 13 RI-One perimeter firewalls, and 6 Windsor firewalls — creating significant blind spots for SIEM/SOC. | Enable syslog on all firewalls minimum at informational (6) level. Target existing collectors: 167.69.34.237 (Aetna) or designated SIEM. PDC DMZ and RI-One perimeter firewalls are highest priority given internet-facing exposure. |
| 3 | Critical | 49 ASA devices have neither a hardware IPS module nor threat-detection enabled. Includes PDC, CME/CMC GLR, and all NET4WIR 3rd-party contexts. | Enable threat-detection basic-threat and threat-detection statistics on all ASA devices immediately — zero-cost, in-software control. Evaluate IPS module deployment for CME/CMC GLR hub firewalls given BPO network exposure. |
| 4 | High | 90 FTD devices with Snort3 are ESSENTIALS-only — IPS rules run in detect-only mode with no blocking. Affects all BPO MPLS (50 devices), all cloud extranet (33 devices), and 2-node DCOM cluster. | Obtain IPS license entitlement for FTD devices on externally-facing segments. Prioritize: (1) MPLSWEST/EAST BPO connections — Snort in blocking mode on 3rd-party MPLS is the highest ROI, (2) Cloud extranet Ashburn/Dallas FTDs. |
| 5 | High | 17 ASA devices negotiate TLS 1.0 — deprecated by RFC 8996 (March 2021) and prohibited under PCI-DSS 4.0. Includes RI-One VPN concentrators (mid-fw-vpn1/2), Windsor VPN concentrators (win-fw-vpn1/2), PDC Aetna/CVTY firewalls, and NET4WIR system context. | Upgrade all affected ASAs to ssl server-version tlsv1.2 and ssl client-version tlsv1.2. Verify TLS 1.0 removal does not break legacy partner VPN clients before cutover — coordinate with 3P connection owners. Target completion before next PCI assessment cycle. |
| 6 | High | NetFlow/flow telemetry is missing on 173 of 183 devices. Stealthwatch/CSNA has no visibility into BPO MPLS, cloud extranet, or NET4WIR 3rd-party traffic. | Configure flow-export on all FTD devices to Stealthwatch collectors. For ASA: add flow-export event-type all to global_policy class-default. This requires only config change — no hardware. Target: 100% of internet-facing and 3rd-party segments. |
| 7 | High | No TLS inspection (SSL decrypt-and-reinspect) deployed anywhere. All HTTPS traffic on 3rd-party connections traverses uninspected regardless of IPS license state. | Evaluate TLS inspection deployment on FTD devices handling web-based 3rd-party connections (Ashburn/Dallas cloud extranet, NET4WIR HTTP flows). Requires SSL license, CA cert deployment, and partner notification. Phase approach by segment risk. |
| 8 | Medium | uRPF (anti-spoofing) is deployed on only 19 of 183 devices. Missing on all PDC, CME/CMC, RI-One perimeter, and Windsor perimeter firewalls. | Enable ip verify reverse-path interface on all external-facing interfaces. Particularly important for RI-One and Windsor perimeter firewalls that terminate internet-routable B2B traffic. |
| 9 | Medium | ASA IPS module mode (inline vs. promiscuous) is not determinable from config. 13 devices show service-module ips keepalive entries but no explicit ips mode configuration. | Audit live IPS module mode via show module and show ips detail on each device. Confirm all modules are in inline (blocking) mode rather than promiscuous (IDS-only). Document mode per device. |
| 10 | Low | HTTP application inspection deployed on only 7 of 86 ASA devices. Remaining 79 inspect DNS, FTP, H.323, SIP — but not HTTP — limiting protocol-level abuse detection on web traffic. | Add inspect http to global_policy class inspection_default on ASA devices handling web-based partner traffic. Use custom policy-map type inspect http to restrict methods, block known-bad patterns, and enforce content type. |

CVS Health — CCD Platform | Information Security | Confidential | June 29, 2026 | Based on firewall running configs — live enforcement state may differ pending config deployment.